SUG Observation Report — 08 Oct 2013

| Observatory: | AJ4CO Observatory | Instrumentation: | DPS on XTFD |
| --- | --- | --- | --- |
| Location: | High Springs, Florida |  | FS-200 on SJ Array |
| Observer: | Dave Typinski |  | Jove Rx on SJ Array |

| Observer On Duty (UTC): | unattended | Body: | Jupiter |
| --- | --- | --- | --- |
| Observer Off Duty (UTC): | unattended | Storm Type: | non-Io-A/C |

| Storm Activity (UTC) |  | CML-III | Io  | Off Axis | DE | JS  |  JE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Begin: | 1050 | 252° | 118° | 14° | +1.74° | –86° | 5.13 AU |
| End: | 1235 | 316° | 132° | 14° |  |  |  |

| Interval |  |  |  |  |  | Fhi | Flo |  |  |  | Drift |  | Rot Rate | TEC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | (UTC) | L | S | N | Pol | MHz | MHz | CML | Io | ML | MHz/m | FB | #/MHz @ F | 1016/m2 |
| 1 | 1050–1235 | X |  |  | RCP | 17 | 25 | 284 | 125 |  |  | X | 1.2 @ 20 |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Interval – Time during which something interesting was observed L, S, N – Indicates presence of L bursts, S bursts, and N events Pol – Dominant circular polarization observed Fhi, Flo – Frequency range of observed emission CML, Io – System III CML and Io phase during noted interval | ML – indicates presence of modulation lanes Drift – Observed modulation lane drift rate in MHz per minute FB – indicates presence of Faraday bands Rot Rate – Observed Faraday rotation rate in rotations per MHz at observing frequency F TEC – Ionospheric vertical total electron count during noted interval as reported by NGDC |
| --- | --- |

Observer Notes

| Data File(s): | AJ4CO-DPS-131008063000.sps AJ4CO-FS-200-131008063000.sps AJ4CO-RSP-UT131008063000.spd |  |
| --- | --- | --- |

Io-CML Phase Plane

RJP Sky View

US-TEC Map

Unavailable due to FedCo shutdown.

| Kp index, 3-hour average: | 2 |
| --- | --- |

CONUS Lightning Activity

| Instrumentation: | DPS on XTFD 30 kHz IF 300 channels, 53 kHz spacing 17–33 MHz |
| --- | --- |

Calibration Scale

Overview Spectrogram (675 px)

| Instrumentation: | FS-200 on SJ Array 30 kHz IF 200 Channels 18–26 MHz |
| --- | --- |

Calibration Scale

Overview Spectrogram (675 px)

| Instrumentation: | Jove Receivers on SJ Array 19.95 and 20.25 MHz |
| --- | --- |

Overview Strip Chart