SUG Observation Report — Date November 30, 2013

| Observatory: | HNRAO | Instrumentation: | FSX Spectrograph |
| --- | --- | --- | --- |
| Location: | Industry, PA |  | JOVE II Receiver |
| Observer: | Jim Brown |  | Dual 20.1/24 MHz dipoles |

| Observer On Duty (UTC): | attended | Body: | Jupiter |
| --- | --- | --- | --- |
| Observer Off Duty (UTC): | attended | Storm Type: | Io-B/Non-Io-B |

| Storm Activity (UTC) |  | CML-III | Io  | Off Axis | DE | JS  |  JE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Begin: | 0601 | 150 | 65.9 |  |  |  |  |
| End: | 0659 |  |  |  |  |  |  |

| Interval |  |  |  |  |  | Fhi | Flo |  |  |  | Drift |  | Rot Rate | TEC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | (UTC) | L | S | N | Pol | MHz | MHz | CML | Io | ML | MHz/m | FB | #/MHz @ F | 1016/m2 |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Interval – Time during which something interesting was observed L, S, N – Indicates presence of L bursts, S bursts, and N events Pol – Dominant circular polarization observed Fhi, Flo – Frequency range of observed emission CML, Io – System III CML and Io phase during noted interval | ML – indicates presence of modulation lanes Drift – Observed modulation lane drift rate in MHz per minute FB – indicates presence of Faraday bands Rot Rate – Observed Faraday rotation rate in rotations per MHz at observing frequency F TEC – Ionospheric vertical total electron count during noted interval as reported by NGDC |
| --- | --- |

Observer Notes

Composed almost entirely of S-Bursts, this event was also populated with S-N burst events. There were periods of strong S burst activity. Activity was between 19 and 23 MHz, and seemed to be centered on 21 MHz. The storm started near 19 MHz and quickly jumped to near 23 MHz and began dropping in frequency ending slightly below 21 MHz. There was brief activity at 20.1 MHz (Radio JOVE) between 0629 UT and 0636 UT.

| Data File(s): |  |  |
| --- | --- | --- |

RJP Sky View

| Instrumentation: | FSX JOVE/24 MHz dual phased dipoles |
| --- | --- |

Calibration Scale

Offset: 330

Gain: 2.5

Overview Spectrogram (675 sweeps)